SoilSense Pro

Smart IoT Soil & Environmental Monitoring System

Complete Project Reference — Hardware + Software + Firebase

Board: ESP32 Dev Module  |  Firebase: soilsense-iot-d1c31  |  By: Ganesh Kumbhar

# 1. Project Overview

SoilSense Pro is a complete smart agriculture IoT system that monitors soil moisture, temperature, and humidity in real-time using an ESP32 microcontroller. Data is sent to Firebase Realtime Database every 5 seconds and displayed on a live web dashboard with AI-based watering predictions, email alerts, and hardware alerts.

## Key Features

| Feature | Technology | Status |
| --- | --- | --- |
| Soil Moisture Monitoring | Capacitive Sensor + GPIO34 | Working |
| Temperature & Humidity | DHT11 + GPIO4 | Working |
| Cloud Database | Firebase Realtime DB | Working |
| Live Dashboard | HTML + Chart.js | Working |
| Login System | Firebase Authentication | Working |
| AI Watering Prediction | JavaScript Algorithm | Working |
| Email Alerts | EmailJS + Gmail | Working |
| LED Alert | External LED + GPIO26 | Working |
| Buzzer Alert | Active Buzzer + GPIO15 | Working |
| CSV Data Export | Browser Download | Working |
| Dark / Light Mode | CSS Theme Toggle | Working |

# 2. Components Required

| Component | Specification | Quantity |
| --- | --- | --- |
| ESP32 Dev Module | ESP32 WROOM-32, 38 pins | 1 |
| Soil Moisture Sensor | Capacitive type, 4-pin, 3.3V | 1 |
| DHT11 Sensor | Temperature + Humidity, 3-pin | 1 |
| LED | Any colour, 5mm external | 1 |
| Active Buzzer | 5V active buzzer, 2-pin | 1 |
| Resistor | 220 ohm (for LED) | 1 |
| Jumper Wires | Male-to-Male / any wire | 10+ |
| USB Cable | Micro USB for ESP32 | 1 |
| Power Supply | 5V USB adapter or laptop | 1 |

# 3. Complete Hardware Wiring

## 3.1 Soil Moisture Sensor (4-pin)

Important: Only 3 of the 4 pins are used. DOUT pin is left unconnected.

| Sensor Pin | Wire Colour | ESP32 Pin |
| --- | --- | --- |
| VCC (Power) | Red | 3V3 |
| GND (Ground) | Black | GND |
| AOUT (Analog Output) | Yellow / Any | GPIO34 |
| DOUT (Digital) | — | NOT CONNECTED (ignore) |

## 3.2 DHT11 Temperature & Humidity Sensor (3-pin)

Pins are ordered left to right when flat side faces you.

| DHT11 Pin | Wire Colour | ESP32 Pin |
| --- | --- | --- |
| VCC (leftmost pin) | Red | VIN (5V) |
| DATA (middle pin) | Green / Any | GPIO4 |
| GND (rightmost pin) | Black | GND |

## 3.3 LED (External)

Identify legs: longer leg = positive (+), shorter leg = negative (-).

| LED Leg | Wire Colour | ESP32 Pin |
| --- | --- | --- |
| + (long leg / positive) | Red | GPIO26 (via 220 ohm resistor) |
| - (short leg / negative) | Black | GND |

Note: If no resistor available, connect directly GPIO26 to LED (+). LED will be brighter but still works.

## 3.4 Active Buzzer (2-pin)

Look for + symbol or longer leg = positive. Active buzzer makes sound with just HIGH/LOW signal.

| Buzzer Pin | Wire Colour | ESP32 Pin |
| --- | --- | --- |
| + (positive / longer leg) | Red | GPIO15 |
| - (negative / shorter leg) | Black | GND |

## 3.5 Complete ESP32 Pin Summary

| ESP32 Pin | Connected To |
| --- | --- |
| 3V3 | Soil Sensor VCC (Red wire) |
| VIN (5V) | DHT11 VCC (Red wire) |
| GND | Soil GND + DHT11 GND + LED (-) + Buzzer (-) — all 4 twisted together |
| GPIO34 | Soil Sensor AOUT (Yellow wire) |
| GPIO4 | DHT11 DATA (Green wire) |
| GPIO26 | LED + long leg (via 220 ohm resistor) |
| GPIO15 | Buzzer + positive leg (Red wire) |

TIP: ESP32 has multiple GND pins. Twist all 4 black wires together and connect to any GND pin — OR use separate GND pins for each component.

# 4. Firebase Setup Steps

| Step | Action |
| --- | --- |
| 1. Create Project | Go to console.firebase.google.com -> Add project -> Name: soilsense-iot-d1c31 |
| 2. Realtime Database | Build -> Realtime Database -> Create Database -> Test mode -> Enable |
| 3. Database Rules | Realtime Database -> Rules tab -> Set .read and .write to true -> Publish |
| 4. Authentication | Build -> Authentication -> Get started -> Anonymous -> Enable -> Save |
| 5. Email/Password Auth | Authentication -> Sign-in providers -> Email/Password -> Enable |
| 6. Register Web App | Gear icon -> Project Settings -> </> Web -> Register app -> Copy firebaseConfig |
| 7. Dashboard Login | Open dashboard.html -> Create account with your email and password |

## Firebase Project Details

| Parameter | Value |
| --- | --- |
| Project ID | soilsense-iot-d1c31 |
| Database URL | https://soilsense-iot-d1c31-default-rtdb.firebaseio.com |
| Auth Domain | soilsense-iot-d1c31.firebaseapp.com |
| Messaging Sender ID | 223496436985 |
| Database Path | /sensor/moisture/{epochTimestamp} |

# 5. Arduino IDE Setup

| Step | Action |
| --- | --- |
| 1. Add ESP32 Board URL | File -> Preferences -> Additional Board URLs -> Add: https://arduino.esp8266.com/stable/package_esp8266com_index.json |
| 2. Install ESP32 Board | Tools -> Board -> Boards Manager -> Search esp32 -> Install by Espressif |
| 3. Select Board | Tools -> Board -> ESP32 Arduino -> ESP32 Dev Module |
| 4. Select Port | Tools -> Port -> Select correct COM port (e.g. COM8) |
| 5. Upload Speed | Tools -> Upload Speed -> 115200 |

## Required Libraries (Tools -> Manage Libraries)

| Library Name | Author | Purpose |
| --- | --- | --- |
| Firebase Arduino Client Library for ESP8266 and ESP32 | Mobizt | Send data to Firebase |
| DHT sensor library | Adafruit | Read DHT11 temperature and humidity |
| Adafruit Unified Sensor | Adafruit | Required by DHT library |
| NTPClient | Fabrice Weinberg | Get current time from internet |

# 6. LED & Buzzer Behaviour

| Condition | LED + Buzzer Action |
| --- | --- |
| System starting up | LED blinks 3 times + Buzzer beeps 2 times (startup test) |
| Soil moisture OK (>30%) | LED quick blink every 5 seconds (heartbeat) + Buzzer silent |
| Soil DRY detected (<30%) | LED turns ON solid immediately |
| Soil DRY (every 30 sec) | Buzzer beeps 3 times + LED blinks 3 times then stays ON |
| Soil recovered (back to OK) | 1 long beep + 2 slow blinks then LED turns OFF |

# 7. Dashboard Features

| Feature | How to Use |
| --- | --- |
| Login | Open dashboard.html -> Enter email + password -> Sign In |
| Live Gauge | Shows real-time soil moisture % with colour zones |
| Real-time Chart | Last 20 readings plotted as line graph |
| Temperature Card | Shows current temp, min, max, avg, heat index |
| Humidity Card | Shows current humidity, min, max, avg |
| AI Prediction | Auto-calculates next watering time after 5+ readings |
| Email Alert | Scroll to bottom -> Enter email -> Save -> Auto-sends when dry |
| Export CSV | Click CSV button in header -> Downloads all readings as Excel file |
| Dark/Light Mode | Click Light/Dark button in header to toggle theme |
| Sign Out | Click Out button in header |

# 8. Firebase Data Structure

Each reading is stored at path: /sensor/moisture/{epochTimestamp}

| Field | Description |
| --- | --- |
| value | Soil moisture percentage (0-100) |
| raw | Raw ADC reading from sensor (0-4095) |
| soil_status | Dry / Moist / Wet |
| temperature | Temperature in Celsius from DHT11 |
| humidity | Air humidity percentage from DHT11 |
| heat_index | Calculated heat index in Celsius |
| temp_status | Cold / Normal / Warm / Hot |
| hum_status | Low / Normal / High |
| alert | true if soil is dry, false if OK |
| timestamp | ISO format: 2024-01-13T20:44:12 |

# 9. EmailJS Configuration

| Parameter | Value |
| --- | --- |
| Service ID | service_rj3sq3y |
| Template ID | template_z90i26a |
| Public Key | qPr-jzIwW1BHiFvsS |
| Email Account | ganeshkumbhar638@gmail.com |
| Trigger Condition | Soil moisture drops below threshold (default 30%) |
| Cooldown | 5 minutes between alerts (no spam) |

# 10. Troubleshooting

| Problem | Cause | Fix |
| --- | --- | --- |
| LED never blinks | Wrong pin or wiring | Check GPIO26 -> long leg -> resistor -> GND |
| Buzzer always ON | Wiring or code issue | Check GPIO15, upload latest firmware |
| Firebase error | Wrong credentials | Check DATABASE_URL has no https:// prefix |
| DHT11 read failed | Wrong pin or loose wire | Check GPIO4 connection, use VIN not 3V3 |
| Soil always 0% | Wrong calibration | Adjust DRY_VAL and WET_VAL in code |
| No data in dashboard | DB rules not set | Set .read and .write to true in Firebase Rules |
| Email not sending | EmailJS not configured | Check Service ID, Template ID, Public Key |
| Compile error stray | Copied chat text into code | Download .ino file directly, do not copy from chat |

SoilSense Pro  |  Smart IoT Project  |  Ganesh Kumbhar  |  soilsense-iot-d1c31